| Name | Duong Minh Tam | No. | T050 | Div/Dept | DSD/ACD/ACT1 | Date：2022/07/01 Job Title | Intern |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Please tick the period | First Month | □W1 □W2 □W3 □W4 |  |  |  |  |  |
|  | Second Month | □W1 □W2 □W3 W4 |  |  |  |  |  |
|  | Third Month | W1 W2 □W3 □W4 |  |  |  |  |  |

- The weekly report aims to help accelerate member’s workspace integration and should be reviewed by mentor on the last working day of the week.

- The new weekly report should be reviewed and signed by mentor and direct supervisor.

Work Experience Record

1. Please describe the tasks and achievements you learned/executed : All of the tasks I learned during this first week are based on the training plan of 2022, below are a brief description of the tasks and what I learned: - How to create setup file, - Trial FSN0FS102A MEM Test-chips

- What are the problems encountered this week? Any actions taken? Any help needed?

3. What are the tasks for next week? Any preparation needed in advance?

| Name (Date) | Mentor | Direct Supervisor |
| --- | --- | --- |
| Alden Duong (2022/07/01) | Charles Le (Signature/Date) | (Signature/Date) |

【Note】

- Please include your afterthought on this week’s training lectures.

- At the end of each report, Manager is to indicate review completion in the blank space.

- What is FSR0FA010A_sim ?

After run ftl2ver => file tb (.vt)

FSR0FA010A_sim is the name of module Testbench.

- Why need Resequence pattern.

- Mode (RESEQ, MASK) ?

ReadOnly_pattern.ftl

Mode = RESEQ => ReadOnly_pattern_tmp.ftl

Mode = MASK => ReadOnly_pattern_reseq.ftl

- What is DCO_Checker used for ?

Check freq of MEM

- How memory work?

- Learn about MBIST and BISR.

- How to test in ram/rom?

The main memory inside a computer is based on two kinds of chip: a temporary, volatile kind that remembers only while the power is on (RAM) and a permanent, nonvolatile kind that remembers whether the power is on or off (ROM).

Memory Built-In Self-Test (MBIST) gives the best solution to test such memories. Built in self-repair (BISR) widely used to test/repair RAM, where each RAM uses dedicated BISR circuit. The BISR feature helps to check Memory BIST logic and memory wrapper interface. Memory testing will become more effective when it adds repair features like Built-In Redundancy Analysis (BIRA) into it.

MBIST is a self-testing and repair mechanism which tests the memories through an effective set of algorithms to detect possibly all the faults that could be present inside a typical memory cell : stuck-at (SAF), transition delay faults (TDF), coupling (CF) or neighborhood pattern sensitive faults (NPSF).

What is Memory Repair?

The memory repair feature diverts faulty areas of memories (e.g. row, column or both) by spare or redundant rows and columns available.

Figure 1 BISR Architecture

Ref: https://www.design-reuse.com/articles/46328/testing-of-repairable-embedded-memories-in-soc.html

MBIST Model

Ref: https://www.einfochips.com/blog/memory-testing-an-insight-into-algorithms-and-self-repair-mechanism/

Ref: T:\Vietnam\DSD\DSD_ACD\Mentor_Doc\Tessent_Frarday_training_MBIST.pdf

# Run MBIST with project FSN0FS102A

## Create setup file:

In previous weeks, use the available setup file to running the DKs (ferc, flre, … ).

In this week, learn how to create setup file.

Setup file to run ferc, flre, … (NOMINAL_VOLTAGE, IO_VOLTAGE, DESIGN, …)

File setup.ftc.fsta

LIBRARY, NOMINAL_VOLTAGE, DESIGN

Base on file sign-off

Base on file APR data in check information FSN0FS102A

## MBIST_insertion

## Compile standard library to tessent library

Path: /home/ftv_training/2022_Internship/tamduong/FSN0FS102A/FE/DFT/MBIST/INPUT/Standard_Cell

| Input |  | libcomp.do.default |
| --- | --- | --- |
| Runfile: |  | run_libcomp |
| Ouput: |  | libcomp.atpglib |

Core library is FSN0U_CRD

- <Project>_ts_pre.cmd

Path: /home/ftv_training/2022_Internship/tamduong/FSN0FS102A/FE/DFT/MBIST/DATA

| <Project>_ts_pre.cmd |  |  |  |
| --- | --- | --- | --- |
| Intput |  | <Project>.v |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | <Project>_ts_pre.cmd |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | dft_cell_select.tcl |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Output: |  | <Project>.v.tmb |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | <Project>_ts_pre.log |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Exc: tessent -shell -dofile FSN0FS102A_ts_pre.cmd -logfile FSN0FS102A_ts_pre.log -replace

- MBIST Insertion

| mbist_insertion.dof | Default |  | # Insert mbist circuit, could be modified based on feature of type and project |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| go_make_add_clock.csh | Default |  | # Script to extract clock period |  |  |  |  |  |
| FSR0FA010A_PLL_src.lis | RD |  | # PLL clock source list for memories |  |  |  |  |  |
| Instance_need_MBIST.dof | Default |  | # Set memories instance option for BIST and BISR |  |  |  |  |  |
| func_debug | Default |  |  |  |  |  |  |  |  |  |  |
| .synnosyp_dc.setup | Default |  |  |  |  |  |  |  |  |  |  |
| rom_content.dof | Create |  | # Rom code file pattern |  |  |  |  |  |  |  |  |
| mem.list | Create |  | # memories list path |  |  |  |  |  |
| register_tdr.dof | Create |  | # set static register static dft signal |  |  |  |  |  |
| add_clock.dof | Create |  | # Clock period file |  |  |  |  |  |

Path:

/home/ftv_training/2022_Internship/tamduong/FSN0FS102A/FE/DFT/MBIST/DATA

| Extract file memories into mem.list |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| - /home/ASICQA/ada/proj/FSN0FS102A/RD/MEMORY_MOD/verilog/*.v |  |  |  |  |  |
| Out | mem.list |  |  |  |  |

| Make clock data file : Prepare file run go_make_add_clock.csh |  |  |  |  |
| --- | --- | --- | --- | --- |
| >> | ./go_make_add_clock.csh |  |  |  |
| Out | add_clock.dof |  |  |  |

File name: gen_dof_file.csh

Path: home/ftv_training/2022_Internship/tamduong/FSN0FS102A/FE/DFT/MBIST/DATA/gen_dof_file.csh

Create:

clock_fix_drc_setting.dof

register_tdr.dof

Instance_need_MBIST.dof

File name: clock_fix_drc_setting.dof

Path: /home/ftv_training/2022_Internship/tamduong/FSN0FS102A/FE/DFT/MBIST/DATA/clock_fix_drc_setting.dof

102A										010A

???

add_primary_inputs PADI_CCLK_SEL/O

add_pin_constraint C1 PADI_CCLK_SEL/O

When running without these 2 lines will have error:

Open vi => tab DRC Violation => double-click on 1 error

Find the reason why is equal = x

PADI_CCLK_SEL = x => so need to add 2 lines

add_primary_inputs PADI_CCLK_SEL/O

add_pin_constraint C1 PADI_CCLK_SEL/O

File name: register_tdr.dof

Path: /home/ftv_training/2022_Internship/tamduong/FSN0FS102A/FE/DFT/MBIST/DATA/register_tdr.dof

File name: Instance_need_MBIST.dof

Path: /home/ftv_training/2022_Internship/tamduong/FSN0FS102A/FE/DFT/MBIST/DATA/Instance_need_MBIST.dof

Figure 3 010A

Figure 4 102A

Difference between auto and on ?

File mbist_insertion.dof

Run mbist_insertion → top_tsdb_outidr/…/FSR0FA010A.vg

Modify file .synopsys_dc.setup

| Output: |  | FSR0FA010A.vg |
| --- | --- | --- |
|  |  | FSR0FA010A.sdc |

| <Project>_ts_post.cmd |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intput |  | FSR0FA010A.vg |  | # Netlist MBIST |  |  |
|  |  |  | <Project>_ts_post.cmd |  | # Do file |  |  |  |  |
|  | Output: |  | FSR0FA010A.v.tstmb |  |  |  |  |  |  |  |  |  |

Reading file T:\Vietnam\DSD\DSD_ACD\ACT1\Tony\Post_mbist_eco.pptx and refer Harry env.

Create file …_ts_post.cmd

- “Tessent tool insert ijtag port to netlist automatically, this increase port the design and is not allowed”

Remove redundant port

File FSN0FS102A.bisr_segment_order

After run project_eco.cmd

Modify sdc

Make file mbist_sdc_tail.sdc

Copy all the set hierarchy setting.

Add clock source macro hierarchy setting.

Add the set case analysis.

Modify the value of X_test_h and clock Selection.

*set_clock_uncertainty: value in file sign-off

*bisr

Make file user_clk.sdc

Copy Memory macro hierarchy setting

Copy all the clock constraint

Copy all the clock constraint

After run

| Output: |  | FSR0FA010A_MBIST.sdc |
| --- | --- | --- |

Fix file Fix_assign.pl

eKMS => eDatabook => FSN0U_CRD => BUF (BUFUCRDX0P4)

Run Pre-STA

DFT_size_cell_only

Modify DMA.sdc: Add the below part at the end of DMA.sdc

Modify MBIST_expand.sdc: Set the propagated clock for the MBIST sdc

# Run Design tool kit check

Check FERC

Check FLEC

Check FSTA

| Run | STA | for both new SDC |  |
| --- | --- | --- | --- |
| >> | FSTA | MBIST_expand.sdc |  |
| >> | FSTA | DMA.sdc |

Run FLEC

Modify flec.cmd

When run flec will have GUI

Check flec.log

Mở Verdi kiểm tra, khi tạo ra bisr, đường tín hiệu của bisr sẽ được tao ra và nối vào mạch, làm mất đường function, nên flec bị fail. => Phải tạo ra một con mux để lựa chọn test mod hay function mod.

ECO netlist

<netlist>.v

<mbist netlist>.v

<netlist>.v.eco

Result =>

Summaries data

PLL_setting ? (.ftlcon)

| >> | ./sum_mem_info_new.pl -gen_db |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Output: |  | FSN0FS102A_MBIST_PAT_config.csv |  |  |  |
| >> | ./sum_mem_info_new.pl -gen_pt |  |  |  |  |
| Output: |  | FSN0FS102A_pt |  |  |  |

Gen-pattern

Modify mbist_pattern_gen.dof

| >> | ./run_gen_pattern |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Output: | ./PAT_GEN_tsdb_outdir_Practice_Data/patterns/FSR0FA010A_gate.patterns_spec_signoff |  |  |  |  |  |  |  |

Resequence pattern

Copy pattern file => wgl_pt

Create file pt_cv.csh

Run script file =>

Error: Name can not more than 32 chars => rename file pattern.

awk -F "_" '{print "mv  " $0 "  "$1"_RPT_"$2"" }' file_name >> rename_pattern_file

| Run script file |  |  |
| --- | --- | --- |
| >> | source | pt_cv.csh |
| Ouput: |  |  |

Recequence pattern

Link the .ftl file

Run fpad

Modify .th

| # | Run gen pattern |  |  |
| --- | --- | --- | --- |
| >> | ./gen_vt.csh |  |  |
| Output: |  | <pattern>.rom |  |
|  |  | <pattern>.vt |  |

Summary Final Report

How to built basic setup file to run ferc, fsta, flre.

Trong project 010A chỉ được làm quen với flow chạy của MBIST là như thế nào, các file hầu như đã có sẵn, chỉ chạy theo flow và hiểu nó hoạt động như thế nào, gồm những bước gì.

Đến với 102A, có được thêm những khái niệm về MBIST là gì, MISR, BISR, BIRA.

Cách tạo ra file setup, các thông số cơ bản cần phải define trong file setup để chạy fsta, ferc, flre.