| Name | Nguyen Anh Huy | No. | T062 | Div/Dept | DSD/ACD/ACT2 | Date：2022/09/30 Job Title | Intern |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Please tick the period | First Month | □W1 □W2 □W3 W4 |  |  |  |  |  |
|  | Second Month | □W1 □W2 □W3 □W4 |  |  |  |  |  |

- The weekly report aims to help accelerate member’s workspace integration and should be reviewed by mentor on the last working day of the week.

- The new weekly report should be reviewed and signed by mentor and direct supervisor.

Work Experience Record

1. Please describe the tasks and achievements you learned/executed : All of the tasks I learned during this first week are based on the training plan of 2022, below are a brief description of the tasks and what I learned: - Introduce fstaH + Task: Read slide, search in eFaraday. + Achievement: Know about STA tool using in Faraday. - Execute fstaH + Task: Read slide. + Achievement: Know how to run STA for pre-layout. - Test Chip: Insert MBIST to the design + Task: Read slide, excel, taking trial in Linux environment. + Achievement: Know detail how to insert MBIST in Test Chip flow.

2. What are the problems encountered this week? Any actions taken? Any help needed? - Issue: - Have trouble while trying to figure command purpose. - Some docs and web can’t access using company mail. - How to solve: - Ask mentor for these command.

3. What are the tasks for next week? Any preparation needed in advance? - Continue running Test Chip flow

| Name (Date) | Mentor | Direct Supervisor |
| --- | --- | --- |
| Huy Nguyen (2022/09/30) | (Signature/Date) | (Signature/Date) |

【Note】

- Please include your afterthought on this week’s training lectures.

- At the end of each report, Manager is to indicate review completion in the blank space.

# Introduce fstaH

fstaH is a Faraday tool kit that help user perform Static Timing Analysis on ASIC designs, this UI employs the commonly-used static timing analysis tool, PrimeTime, as the analysis engine.

Input of fstaH include

- Setup.ftc.fsta

- Netlist

- SDC:

- Faraday Technology File

- RC Extraction

- Ccsndb

- MSV project (.cpf: Cadence power format, .upf: unified power format, .mmmc: multi mode multi corner file during the physical design gives the analysis of the design over varied modes & corners.)

- .cpf file is Cadence Power format, which describe the features of the multi-voltage project. This is only required if users run fstaH with the CPF-in flow syntax.

- Fsta.control: After fstaH parser will generate a file, which contain information for fstaH to run

# execute fsta

Working directory: /home/FSR0FA010A_FE/FSR0FA010A_Kyle/FE/Fsta/Pre_STA/Dtb

# mbist_test chip

Working directory: /home/FSR0FA010A_FE/FSR0FA010A_Kyle/FE/DFT/MBIST/

## input data for project

My mentor gave me a link which contain data for this project

## compile standard library to tessent library

Woriking directory: /home/FSR0FA010A_FE/FSR0FA010A_Kyle/FE/DFT/MBIST/INPUT/Standard_Cell

| INPUT |  |
| --- | --- |
| libcomp.do.default | # file contain command to translate models to library |
| EXECUTE |  |
| run_libcomp | # file contain directory to source to environment |
| OUTPUT |  |
| libcomp.atpglib | library file |

## mbist insertion

Working directory: /home/FSR0FA010A_FE/FSR0FA010A_Kyle/FE/DFT/MBIST/MBIST_insertion

### FSR0FA010A_ts_pre.cmd

| INPUT |  |
| --- | --- |
| <Project>.v | # Netlist |
| <Project>_ts_pre.cmd | # Dofile: Insert or gate to test_h signal to control clock, could be modified base on each project |
| dft_cell_select.tcl | # Default |

Execute command: tessent -shell -dofile FSR0FA010A_ts_pre.cmd -logfile FSR0FA010A_ts_pre.log –replace

| OUTPUT |  |
| --- | --- |
| <Project>.v.tmb | # Netlist after running this step |
| <Project>_ts_pre.cmd.log | # log file include contain from terminal |

### MBIST Insertion

To start running Mbist insertion, user have to prepare input data first

| INPUT |  |  |
| --- | --- | --- |
| mbist_insertion.dof | Default | # Insert mbist circuit, could be modified based on feature of type and project |
| go_make_add_clock.csh | Default | # Script to extract clock period, output is add_clock.dof |
| FSR0FA010A_PLL_src.lis | RD | # PLL clock source list for memories, use script to generate .dof file from this file |
| Instance_need_MBIST.dof | Create | # Set memories instance option for BIST and BISR |
| func_debug | Default | # run to debug function of the design |
| .synnosyp_dc.setup | Default |  |
| clock_fix_drc_setting.dof | Create | # add pin and pin constraints to memory |
| rom_content.dof | Create | # Rom code file pattern |
| mem.list | Create | # memories list path |
| register_tdr.dof | Create | # set static register static dft signal |
| add_clock.dof | Create | # Clock period file |

Mem.list, add_clock.dof, register_tdr.dof, Instance_need_MBIST.dof, rom_content.dof have been generated by user.

Execute file: run_mbist

#### Create mem.list

Working directory: /home/FSR0FA010A_FE/FSR0FA010A_Kyle/FE/DFT/MBIST/MBIST_insertion/script/

To create mem.list, user have to extract all memory file into mem.list

#### Create add_clock.dof

This file have script to generate this file, to generate add_clock.dof, user have to put script file and FSR0FA010A_PLL_src.lis at the same directory.

#### Create clock_fix_drc_setting.dof

At the beginning, there isn’t script to create clock_fix_drc_setting.dof, so I have to make this file by hand.

To create this file, I have to create a script to generate clock_fix_drc_setting.dof. This script and MBIST_mem.list have to be same directory to create this file.

#### Create register_tdr.dof

#### Create Instance_need_MBIST.dof

#### Insert MBIST first time

First time running insert MBIST, tool will detech that there is a missing file while running mbist_insertion.dof. So the tool will stop execute and generate dft_spec.org file

#### Create rom_content.dof

#### Pre-insert MBIST

Run MBIST insertion step again to insert to design after create rom_content.dof