| Name | Le Nhat Minh | No. |  | Div/Dept | DSD/ACD/ACT1 | Date：2023/09/20 Job Title | Engineer |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Please tick the period | First Month | □W1 □W2 □W3 □W4 |  |  |  |  |  |
|  | Second Month | □W1 □W2 □W3 W4 |  |  |  |  |  |
|  | Third Month | □W1 □W2 □W3 □W4 |  |  |  |  |  |

- The weekly report aims to help accelerate member’s workspace integration and should be reviewed by mentor on the last working day of the week.

- The new weekly report should be reviewed and signed by mentor and direct supervisor.

Work Experience Record

1. Please describe the tasks and achievements you learned/executed : - Testchip: MBIST insertion

2. What are the problems encountered this week? Any actions taken? Any help needed? - Not yet

3. What are the tasks for next week? Any preparation needed in advance? - Continue with Test Chip flow

| Name (Date) | Mentor | Direct Supervisor |
| --- | --- | --- |
| Le Nhat Minh (2023/09/20) | (Signature/Date) | (Signature/Date) |

- Compile standard library to tessent library

(/home/ftv_training/AC_training/Intern_Mar_2023/Minh_Le/FSN0FS126A/FE/DFT/MBIST/INPUT)

- Input file: libcomp.do.default

- Run file: run_libcomp

- Output file: libcomp.atpglib

- MBIST Insertion

(/home/ftv_training/AC_training/Intern_Mar_2023/Minh_Le/FSN0FS126A/FE/DFT/MBIST/DATA_20_09)

- Input file:FSN0FS126A.v (RD)

FSN0FS126A_0P9V_PLL_src.lis (RD)

dft_cell_select.tcl (define cell type for MBIST insertion)

.synopsys_dc.setup (for synthesis)

register_tdr.dof (set static register static dft signal all_test = 1 and MOE)

mem.list (list verilog file of memory)

mbist_insertion.dof (insert MBIST circuit)

Instance_need_MBIST.dof (method test BIST on)

clock_fix_drc_setting.dof ( add constraint for MOE pin)

func_debug (for synthesis)

- Run file: run_mbist_insert

- Output file: FSN0FS126A.vg

- Results:

clock_fix_drc_setting

Instance_need_MBIST

add_clock

mem list

mbist_insertion

run_file

Output: Netlist after insert MBIST FSN0FS116A.vg

Log file:

- Can’t generated file.vg if command option in file mbist_insertion.dof

on option

Results:

off option

Results:

Run file have source link project environment

After insert MBIST circuit, perform step ts_post.cmd (connect JTAG from module to port of chip, insert MUX to select MOE signal of memory and replace clock gatting cell by ICG)

Input files: FSN0FS126A.vg, FSN0FS126A_ts_post.cmd, dft_cell_select.tcl, Fix_assign.pl (connect buffer from port chip)

Output files: FSN0FS126A.v.tmb, FSN0FS126A.v.fixass

Run files: run_mbist_post.csh

Results:

Connect JTAG pin from module to port of chip and insert MUX to control test_mode and function mode

Insert AND gate (synchronized X_A1 and X_test_h)

Repalce gatting cell by ICG

- When X_test_h enable (X_test_h = 1)  Test_H_INV = 0  Test mode  Value of TSTMB_MUX_ijtag_so/O = mux2pad_DOA1.

- When X_test_h unenable (X_test_h = 0)  Test_H_INV = 1 Function mode  Value of TSTMB_MUX_ijtag_so/O = ijtag_so

FLEC

- Check with ts_post netlist, in this case not insert AND gate

Input file: flec.cmd (dofile), golden netlist (RD), FSN0FS126A.v.tmb (netlist after insert MBIST)

Results: Not equivalence

- Check with netlist fix assign (not insert AND gate)

Results: Not equivalence

Signal IJTAG is connected with port external using buffer. However, X_A1 direc connection with ijtag_reset.

- Check with netlist after fixass

Input file: flec.cmd (dofile), golden netlist (RD), FSN0FS126A.v.fixass (netlist after insert MBIST), setup.ftc, gen_no_translate.csh

Results:

- Don’t need eco to fix non equivalence checking