| Name |  | No. |  | Div/Dept | DSD/ACD/ACT | Date：2024/03/29 Job Title | Intern |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Please tick the period | First Month | □W1 W2 □W3 □W4 |  |  |  |  |  |
|  | Second Month | □W1 □W2 □W3 □W4 |  |  |  |  |  |
|  | Third Month | □W1 □W2 □W3 □W4 |  |  |  |  |  |

- The weekly report aims to help accelerate member’s workspace integration and should be reviewed by mentor on the last working day of the week.

- The new weekly report should be reviewed and signed by mentor and direct supervisor.

Work Experience Record

1. Please describe the tasks and achievements you learned/executed : All of the tasks I learned during this first week are based on the training plan of 2024, below are a brief description of the tasks and what I learned: - Understanding Test chip flow - Reading basic about MBIST concept - Practice to insert MBIST to the netlist

2. What are the problems encountered this week? Any actions taken? Any help needed? - Have trouble while trying to figure command purpose - Don’t understand clearly about Rom code file pattern

3. What are the tasks for next week? Any preparation needed in advance? - Continue running Test Chip flow

| Name (Date) | Mentor | Direct Supervisor |
| --- | --- | --- |
| Ho Nguyen Hoang 27/07/2024 | (Signature/Date) | (Signature/Date) |

# I. Test chip flow

# 2. MBIST concept

## a. BIST (Built-In Self-Test):

- BIST is a DFT technique used in VLSI to test the functionality of integrated circuits (ICs) during the manufacturing process and throughout the product's life.

- This circuitry can be designed to test various aspects of the IC, such as logic gates, memory and communication interfaces.

## b. MBIST (Memory Built-In Self-Test)

- MBIST architecture:

+ Controller: Provide control signal to address/background generators

+ Comparator: CP data read from memory during a read operations with the golden data

+ Background Pattern generator: Generate data for memory to perform the write operations, which controlled by the controller

_Binary/Gray counter

_LSFR

+ Address generator:

+ FSM: Control read and write signals

+Mux

_Test_mode: select the address and data generators

_Normal_mode: select the address and data from the processor

| Memory RAM test | Memory ROM test |
| --- | --- |
| March Test Pattern March C+ Algorithm Checkerboard algorithm | MISR (Multiple Input Shift Register) Algorithm: (Reduce test logic by using multiple bit streams to create a signature.) |

## c. MBIST insertion

- Compile standard library to tessent library

Working directory: /INPUT/Standard_cell

| Input | libcomp.do.default |
| --- | --- |
| Output | libcomp.atpglib |
| Run file | run_libcomp |

- Mbist insertion

+ mbist_insertion.dof

Working directory: /DATA

| Input | <Project>_PLL_src.lis <Project>.v <Project>_MBIST_inst.lis go_make_add_clock.csh func_debug .synnosyp_dc.setup dft_cell_select.tcl mbist_insertion.dof Instance_need_MBIST.dof mem.list register_tdr.dof add_clock.dof | PLL clock source list for memories # Pure netlist # MEM instances need MBIST # Script to extract clock period # For synthesis # For synthesis # Define cell type for MBIST insertion, it can be modified for design intention # Insert mbist circuit, could be modified based on feature of type and project # Set memories instance option for BIST and BISR # memories list path # set static register static dft signal # Clock period file |
| --- | --- | --- |
| Output | <Project>.vg <Project>.sdc | # Post MBIST netlist # MBIST SDC |
| Run file | run_mbist.csh |  |

Step 1: >> mem.list

Step 2: ./go_make_add_clock.csh

>> add_clock.dof: Define the clock applied to primary input that is used for MBIST

Step 3: ./go_make_clock_fix_drc.csh

>> clock_fix_drc_setting.dof

Step 4: ./go_make_register_tdr.dof

>> register_tdr.dof

register_static_dft_signal_name: create a new dft_signal and specify its behavior -default_value_in_all_test: all signals use the specified default value

add_dft_control_points: insert the controlling logic during DFT insertion –

dft_signal_source_name: source the pins during test mode.

Step 5: ./go_make_Instance_need_MBIST.dof

>> Instance_need_MBIST.dof

Step 6:

>> func_debug

>>./synopsys_dc.setup

Step 7: >> mbist_insertion.dof

Step 8: ./run_mbist.csh

>> top_tsdb_outdir

+<Project>_ts_post.cmd

Working directory: /MBIST_Post

| Input | <Project>.vg <Project>_ts_post.cmd dft_cell_select.tcl Fix_Assign.pl | # Post MBIST netlist # Do file for post MBIST ECO # Define cell type for MBIST insertion, it can be modified for design intention # Script to fix assign command in gate netlist (connect buffer from port chip) |
| --- | --- | --- |
| Output | <Project>.v.fixass | # Post MBIST netlist after replacing assign command |
| Run file | run_mbist.csh |  |

Step 1: >> dft_cell_select.tcl

Step 2: >> <Proj>_ts_post.cmd

Step 3: Run post MBIST ECO

./run_mbist.csh

>> <Proj>.v.tstmb

>> <Proj>.v.fixass

+ ECO to fix "Non-equivalent signal" when running FLEC for pure netlist and post MBIST netlist

Working directory: /ECO

| Input | <PROJ>_MBIST_inst.lis <PROJ>.v <PROJ>.v.fixass gen_no_translate.csh gen_mbist_constraint.pl flec.cmd batchResults.tcl flec.log <PROJ>.mod <PROJ>.v.fixass setup.ftc make_eco.csh | # List of memory needs MBIST # Pure netlist # Post MBIST netlist after replacing assign command # Script to generate "library_mem.list" and "add_no_translate.dof" # Script to generate "mbist_constraint.dof" # Command file of FLEC, need to update for each project # Log file of FLEC # List *.v file # Post MBIST netlist after replacing assign command # Design Kit setup file # Generate "eco_reconnect_bisr.tcl" file |
| --- | --- | --- |
| Output | <PROJ>.v.feco0 | # Post MBIST netlist after replacing assign command |
| Run file | run_flec.csh run_feco.csh |  |

Step 1: RUN FLEC

Working directory: /ECO/FLEC_post_dft

./gen_no_translate.csh

>>library_mem.list

>>add_no_translate.dof

./gen_mbist_constraint.pl

>> mbist_constraint.dof

Update flec.cmd

./run_flec.csh

>>flec.log

Step 2: Run ECO

Working directory: /ECO/ECO_post_dft

./make_eco.csh

>> eco_reconnect_bisr.tcl

./run_feco.csh

>>  <Proj>.v.feco0

Step 3: Re-run FLEC